# RideProtect-RV Quick Start

# RideProtect-RV — Quick-Start Guide

Goal: From zero to blinking an LED in under 5 minutes.

## 1. Prerequisites

sudo apt-get install -y gcc-riscv64-linux-gnu verilator yosys bsdmainutils make

## 2. Build and Run

git clone <repo-url> rideprotect-rv
cd rideprotect-rv
make firmware          # 2 seconds
make sim               # 10 seconds → see test results
make syn               # 30 seconds → resource estimate

## 3. Write Custom Firmware

Edit sw/boot/test_prog.c. Replace main() with:

int main(void) {
    GPIO_DIR = 0x0F;                 // LEDs as outputs
    UART_DIV = 27; UART_CTRL = 0x03; // 115200 baud
    uart_puts("Blinking!\n");
    while (1) {
        GPIO_DATA_OUT = 0x0F;        // LEDs on
        delay(500000);
        GPIO_DATA_OUT = 0x00;        // LEDs off
        delay(500000);
    }
}

make firmware && make sim

## 4. Deploy to FPGA

make pnr-arty         # Vivado P&R (timing MET: WNS=2.869ns)
# Program build/soc_top.bit via Vivado Hardware Manager
screen /dev/ttyUSB1 115200  # Should show "Blinking!"

## 5. Key Addresses (RAM Map)

0x0000_0000  SRAM (8 KB code+data)
0x2000_0000  GPIO         0x2000_1000  UART
0x2000_2000  Timer        0x2000_3000  INTC
0x2000_4000  Watchdog     0x2000_5000  SPI
0x2000_6000  I2C          0x2000_7000  CAN
0x2000_8000  DMA          0x2000_9000  PWM Advanced
0x2000_A000  RTC          0x2000_B000  Debug/JTAG mailbox

## 6. Quick Make Targets

| Command | What |
| --- | --- |
| make firmware | Build firmware |
| make sim | Build + run simulation |
| make lint | Verilator lint check |
| make syn | Yosys synthesis |
| make clean | Remove build artifacts |

## 7. Where to Find Help

| Document | File |
| --- | --- |
| User Manual | docs/RideProtect_RV_User_Manual.md |
| Project Report | docs/RideProtect_RV_Project_Report.md |
| Firmware Guide | docs/firmware_guide.md |
| Integration Guide | docs/integration_guide.md |
| Setup Guide | docs/setup_guide.md |
| Register Reference | docs/bus_map.md |
| Architecture | docs/architecture.md |