Table of Contents

# VexRiscv Integration Notes

## Status: COMPLETE (as of Phase 5)

The CPU slot transitioned from PicoRV32 to VexRiscv. This document records the integration details for reference.

## Current CPU: VexRiscv (wb_cpu_wrapper.v)

- RV32IMC + Privileged ISA (compressed instructions enabled)

- Dual Wishbone B4 buses: iBus (instruction fetch) and dBus (data access)

- iBus includes 4 KB 1-way instruction cache (InstructionCache sub-module)

- dBus has direct memory access (no data cache)

- iBus and dBus are merged into a single shared Wishbone bus by wb_cpu_wrapper.v

- dBus requests get priority over iBus requests in the wrapper

## Wrapper: wb_cpu_wrapper.v vs. Legacy cpu_slot.v

| Aspect | wb_cpu_wrapper.v (active) | cpu_slot.v (legacy, in rtl/legacy/) |
| --- | --- | --- |
| CPU | VexRiscv | PicoRV32 |
| Bus | Dual (i + d), merged | Single Wishbone |
| IRQ | ext_irq input | Single irq line |
| Debug | JTAG via external TAP | None |

The cpu_slot.v file has been moved to rtl/legacy/ as a historical reference.

## VexRiscv Configuration (as instantiated in wb_cpu_wrapper.v)

- RV32IMC with compressed instructions

- Wishbone B4 master (not AXI4)

- Single-cycle multiply

- Barrel shifter (not light)

- External interrupt controller (via the global wb_interrupt_controller)

- JTAG debug (via external jtag_tap module)

- No PLIC (peripheral interrupt controller) — interrupts are OR-ed through wb_interrupt_controller

## Toolchain

Both cores use the same RISC-V GCC toolchain. VexRiscv supports -march=rv32imc for compressed instructions (currently using rv32im_zicsr; switching to rv32imc_zicsr would reduce firmware size by ~15-20%).

## Address Map

See bus_map.md for the complete memory map.